Project #5 - North Dakota Pipeline Water Quality Analysis

Prepared by: Rashid Hamid, Abbas Bashir, and Abdi Aden

Course: CIVE 202 - Civil Engineering Analysis II

Project: Project #5 - North Dakota Pipeline Water Quality Analysis

# Purpose

The technical executive summary below provides an overview of key findings from the Python water quality analysis conducted to determine any change in water quality from Lake Sakakawea, the Sheyenne River near Lisbon, and Lake Ashtabula. TDS, sulfate, calcium, magnesium, and pH are of key focus in this analysis.

# Methods

External dataset named Project5.csv for water quality data imported into Python.

Converting result measurements into numeric values and removing missing values for key analysis columns.

Filtering down the water quality dataset to the required major water quality parameters according to client.

Calculating mean, median, min, max and count for each location and parameter.

Plotting boxplot graphs, an average TDS graph and comparative scenario graph.

Taking care of creation of mass loading function and TDS mixing function.

# Key Technical Findings

The average TDS rose from 394.60 mg/L in Lake Sakakawea to 783.32 mg/L in Sheyenne River - Lisbon and 902.90 mg/L in Lake Ashtabula. This is indicative of higher concentrations of dissolved solids in the assessed system.

There was an increase in sulfate from 158.64 mg/L in Lake Sakakawea to 363.98 mg/L in Lake Ashtabula. There was also an increase in magnesium from 19.89 mg/L to 60.34 mg/L in the above analysis.

The scenario testing model predicted mixed TDS concentrations of 642.86 mg/L, 676.92 mg/L, and 706.45 mg/L for the low flow, base case, and high flow examples, respectively.

Table 1. Key Summary Statistics

| Location | Parameter | Mean | Median | Min | Max | Count |
| --- | --- | --- | --- | --- | --- | --- |
| Lake Sakakawea | Calcium | 49.45 | 49.50 | 41.00 | 56.10 | 78 |
| Lake Sakakawea | Magnesium | 19.89 | 19.70 | 17.50 | 25.60 | 78 |
| Lake Sakakawea | Sulfate | 158.64 | 159.00 | 133.00 | 224.00 | 78 |
| Lake Sakakawea | Total dissolved solids | 394.60 | 394.50 | 341.00 | 476.00 | 78 |
| Lake Sakakawea | pH | 8.22 | 8.25 | 7.24 | 9.40 | 3485 |
| Sheyenne River - Lisbon | Calcium | 85.27 | 85.80 | 43.70 | 134.00 | 87 |
| Sheyenne River - Lisbon | Magnesium | 52.61 | 52.50 | 24.60 | 91.00 | 87 |
| Sheyenne River - Lisbon | Sulfate | 300.95 | 295.00 | 114.00 | 686.00 | 87 |
| Sheyenne River - Lisbon | Total dissolved solids | 783.32 | 768.00 | 337.00 | 1540.00 | 87 |
| Sheyenne River - Lisbon | pH | 8.29 | 8.28 | 7.28 | 11.98 | 172 |
| Lake Ashtabula | Calcium | 74.74 | 75.70 | 48.80 | 128.00 | 292 |
| Lake Ashtabula | Magnesium | 60.34 | 58.15 | 29.80 | 92.40 | 292 |
| Lake Ashtabula | Sulfate | 363.98 | 327.00 | 156.00 | 686.00 | 291 |
| Lake Ashtabula | Total dissolved solids | 902.90 | 842.00 | 436.00 | 1540.00 | 283 |
| Lake Ashtabula | pH | 8.51 | 8.56 | 6.81 | 9.28 | 1388 |

# Most Concerning Parameters

TDS and sulfate seem to be the two most critical parameters, since there is a marked increase in these parameters, which are directly associated with the problem of dissolved solids, scaling, and taste issues in water. Magnesium and calcium levels have also shown an increase.

# Decision Making and Limitations

The analysis was done using select points and variables to ensure that the notebook remained clean and followed the client’s requirements. The removal of missing values ensures that the calculations are accurate but does not guarantee that the missing values are not valuable since they may be random. The TDS mixing model assumes perfect mixing, which may not reflect hydraulic behavior in rivers or lakes.

# Conclusion

The results indicate that the amount of dissolved components increases through the system under evaluation. More work is required to verify the flow rates from the data used, incorporate additional sampling points for canal water samples if possible, and assess ways to improve water quality.