Scope of Work

Project: North Dakota Pipeline Water Quality Analysis
Client: Garrison Diversion Project, Red River Valley Water Supply Project (RRVWSP)
Prepared by: Abdi Aden, Rashid Hamid, Abbas Bashir - CIVE 202 Engineering Team
Date: 05/01/26

Project Goals

The primary focus of this research paper shall be the evaluation of water quality status in the McClusky Canal system, as well as the possible influence of water transfer between Lake Sakakawea and the Sheyenne River, located near Lake Ashtabula. This part of the research shall involve identification of any water quality changes, assessment of associated risks, and evaluation of whether implementation of the proposed system will result in a negative impact on water quality downstream.

Also, there is a need to develop a mass balance model that predicts TDS in the mixture of canal water with the Sheyenne River.

Project Tasks

Task 1: Review Datasets and Metadata

Responsible Party: Abdi Aden

An examination of the metadata associated with the USGS Water Quality data will provide insights into the definitions of each of the variables within the dataset. The data includes measurements from various locations on the McClusky Canal.

This task includes:

- Identifying key water quality parameters

- Understanding units and measurement methods

- Verifying data consistency and structure

- Reviewing canal location data

Software Used: Python (pandas)

Task 2: Select Water Quality Parameters for Analysis

Responsible Party: Abdi Aden

Water quality variables will be selected based on their relevance to environmental impact and client requirements.

Major Cations:

- Calcium (Ca)

- Magnesium (Mg)

- Sodium (Na)

- Sodium Adsorption Ratio

Major Anions:

- Chloride (Cl)

- Sulfate

- Carbonate

- Bicarbonate

Other Parameters:

- Total Dissolved Solids (TDS)

- Temperature

- pH

- Total Hardness

These variables are selected because they directly influence water quality, environmental health, and suitability for downstream use.

Task 3: Data Cleaning and Processing

Responsible Party: Rashid Hamid

The dataset will be cleaned and prepared for analysis to ensure accuracy and consistency.

This includes:

- Removing or handling missing values

- Filtering relevant canal locations

- Selecting required variables

- Ensuring consistent units and formats

- Organizing data by location

This step ensures the dataset is reliable and ready for analysis.

Software Used: Python (pandas, NumPy)

Task 4: Calculate Summary Statistics

Responsible Party: Abbas Bashir

Summary statistics will be calculated for each water quality parameter at each location.

This includes:

- Mean

- Median

- Maximum

- Minimum

These statistics will be used to compare water quality across locations and identify potential problem areas.

Task 5: Create Data Visualizations

Responsible Party: Abdi Aden

Visualizations will be created to support analysis and communicate results clearly.

This includes:

- Boxplots for at least four water quality parameters

- Comparison of parameter distributions across canal locations

All plots will include:

- Titles

- Labeled axes

- Clear formatting

Software Used: Python (matplotlib, seaborn)

Task 6: Perform Mass Balance Analysis

Responsible Party: Abdi Aden, Rashid Hamid

A mass balance model will be developed to calculate TDS concentration in Lake Ashtabula after mixing.

Mass Balance Equation:
QpipeCpipe + QSheyenneCSheyenne = QAshtabulaCAshtabula

This task includes:

- Using pipe flow rate (6 cfs)

- Extracting Sheyenne River flow rate from dataset

- Calculating resulting TDS concentration

- Creating a dynamic model for multiple scenarios

This allows evaluation of how different flow conditions affect downstream water quality.

Task 7: Analyze and Interpret Results

Responsible Party: Abbas Bashir

The results will be analyzed to determine:

- Whether water quality degrades along the canal

- Which parameters are most concerning

- How mixing affects downstream water quality

- Potential environmental impacts

The analysis will also include recommendations for improving water quality.

Task 8: Prepare Final Deliverables

Responsible Party: Rashid Hamid

The following deliverables will be completed:

- Python code file (.ipynb)

- Annotated Code Document (ACD)

- Scope of Work (SOW)

- Gantt Chart

- Engineering Timesheet

- Technical Report

- Raw Data Files

- GitHub repository with README

Project Deliverables

All deliverables will be submitted by May 8, 2026 at 5:00 PM CT.

| Deliverable | Files Included | Due Date |
| --- | --- | --- |
| GitHub Repository | Code, data, README | 05/08/2026 |
| Python Code | Jupyter Notebook (.ipynb) | 05/08/2026 |
| Annotated Code Document | ACD.docx | 05/08/2026 |
| Scope of Work | SOW.docx | 05/08/2026 |
| Gantt Chart | PDF | 05/08/2026 |
| Timesheet | Engineering Timesheet | 05/08/2026 |
| Technical Report | Final Report.docx | 05/08/2026 |

Expected Outcome

By the end of the project, the client will have knowledge about the water quality situation in the McClusky Canal and its possible impacts if the pipeline system is implemented. The study will reveal how water mixing plays a role in TDS and what major issues the water quality faces. This project will show how Python can be used as a useful tool for analyzing situations in environmental engineering.