Project #5 - North Dakota Pipeline Water Quality Analysis

Prepared by: Rashid Hamid, Abbas Bashir, and Abdi Aden

Course: CIVE 202 - Civil Engineering Analysis II

Project: Project #5 - North Dakota Pipeline Water Quality Analysis

# Project Purpose

The research focuses on the change in the water quality as the water flows from Lake Sakakawea to the Sheyenne River and Lake Ashtabula. The research seeks to determine whether the water quality is enriched with more dissolved substances at downstream sites.

# What Was Analyzed

The project incorporated water quality information using water quality measurements, including total dissolved solids, sulfates, calcium, magnesium, and pH levels. Such measurements will help indicate whether the mineral content is increasing or the water hardness is increasing.

# Main Results

Total dissolved solids went up from approximately 395 mg/L in Lake Sakakawea to 903 mg/L in Lake Ashtabula. This indicates that there was more dissolution in the water in the second lake compared to the first lake in this study.

There was also an increase in sulfate concentration from approximately 159 mg/L to 364 mg/L. Thus, sulfate and total dissolved solids are two of the key indicators.

# TDS Mixing Model

An simple mixing model was developed for calculating the concentration of total dissolved solids that would occur if the water from the canal were to mix with the river water. This model can be modified through the input of various values of the flow rate and dissolved solids.

Table 1. Example TDS Mixing Scenarios

| Scenario | Pipe Flow (cfs) | Pipe TDS (mg/L) | River Flow (cfs) | River TDS (mg/L) | Mixed TDS (mg/L) |
| --- | --- | --- | --- | --- | --- |
| Low Flow | 6 | 550 | 15 | 680 | 642.86 |
| Base Case | 6 | 600 | 20 | 700 | 676.92 |
| High Flow | 6 | 650 | 25 | 720 | 706.45 |

# What This Means

These findings imply that care must be taken when monitoring the water quality since the levels of dissolved solids and sulfate tend to increase within the various sites. However, this does not imply that the project should not proceed; rather, it implies that the water quality effects should be considered beforehand.

# Important Limits

In this case study, selected sites and water quality parameters are used. Actual water distribution networks might have complications due to variations of flow, mixing, season, and weather conditions, affecting the quality of water. Additional data and engineering studies will enhance the reliability of the final conclusion.